Project Setup:
The_Fortress_OS/

├── config.yaml

├── requirements.txt

├── main.py

├── modules/

│   ├── __init__.py

│   ├── comms.py

│   ├── sentinel.py

│   ├── caretaker.py

│   ├── sustainer.py

│   └── mcp3008.py

└── dashboard/

└── app.py

Configuration & Dependencies:

requirements.txt:
opencv-python-headless>=4.8.0

ultralytics>=8.0.0

paho-mqtt>=1.6.1

requests>=2.31.0

streamlit>=1.25.0

pyyaml>=6.0

spidev>=3.6  # Comment this out if testing on Windows

RPi.GPIO>=0.7.1 # Comment this out if testing on Windows

config.yaml:

system:

mode: "SIMULATION" # Change to "PRODUCTION" on Raspberry Pi

mqtt:

broker: "localhost"

port: 1883

topics:

security: "home/security"

automation: "home/automation"

biosphere: "home/biosphere"

location:

lat: 29.3759  # Kuwait City

long: 47.9774

camera:

index: 0

model_path: "yolov8n.pt"

gpio:

# BCM Pin Numbers

relay_lock: 17

relay_shaker: 27

servo_feeder: 18

stepper_pins: [22, 23, 24, 25]

The Modules (The Logic):

modules/comms.py (MQTT Handler):
import paho.mqtt.client as mqtt

import time

def init_mqtt(broker, port=1883):

client = mqtt.Client("Fortress_Brain")

try:

client.connect(broker, port, 60)

client.loop_start()

print(f"[MQTT] Connected to {broker}")

return client

except Exception as e:

print(f"[MQTT] Connection Failed: {e}")

return None

def publish(client, topic, message):

if client:

client.publish(topic, message)

# print(f"[MQTT Tx] {topic}: {message}") # Uncomment for debug

modules/sentinel.py (Security & Vision):
import cv2

from ultralytics import YOLO

import time

from modules import comms

def run(config, client):

print(">> Sentinel Module: ONLINE")

# Load Model (Downloads automatically on first run)

model = YOLO(config['camera']['model_path'])

cap = cv2.VideoCapture(config['camera']['index'])

# Threat Classes (COCO dataset indices)

# 43: Knife, 67: Cell Phone (for testing), 0: Person

THREAT_CLASSES = [43, 67]

while True:

ret, frame = cap.read()

if not ret:

time.sleep(1)

continue

# Inference

results = model.predict(frame, conf=0.5, verbose=False, imgsz=320)

threat_detected = False

for r in results:

for box in r.boxes:

cls = int(box.cls[0])

if cls in THREAT_CLASSES:

threat_detected = True

label = model.names[cls]

# --- ACTION PROTOCOL ---

msg = f"THREAT DETECTED: {label}"

print(f"!!! [SENTINEL] {msg} !!!")

comms.publish(client, config['mqtt']['topics']['security'], "LOCKDOWN")

# [HARDWARE] Uncomment on Pi

# GPIO.output(config['gpio']['relay_lock'], GPIO.HIGH)

if not threat_detected:

# Heartbeat to dashboard

comms.publish(client, config['mqtt']['topics']['security'], "SAFE")

# Optional: Show feed (Disable on headless Pi)

# cv2.imshow('Sentinel Eye', results[0].plot())

# if cv2.waitKey(1) == ord('q'): break

time.sleep(0.5) # Throttle to save CPU

modules/caretaker.py (Automation & Faith):
import requests

import datetime

import time

from modules import comms

def get_prayer_times(lat, long):

try:

url = f"http://api.aladhan.com/v1/timings?latitude={lat}&longitude={long}&method=4"

res = requests.get(url, timeout=5).json()

return res['data']['timings']

except:

return None

def run(config, client):

print(">> Caretaker Module: ONLINE")

# State tracking

prayer_times = get_prayer_times(config['location']['lat'], config['location']['long'])

last_check = datetime.datetime.now().day

while True:

now = datetime.datetime.now()

current_time_str = now.strftime("%H:%M")

# Refresh API daily

if now.day != last_check:

prayer_times = get_prayer_times(config['location']['lat'], config['location']['long'])

last_check = now.day

# --- Faith Logic ---

if prayer_times:

for prayer, p_time in prayer_times.items():

if current_time_str == p_time:

print(f"[CARETAKER] It is {prayer} time. Muting media.")

comms.publish(client, config['mqtt']['topics']['automation'], f"PRAYER_{prayer.upper()}")

time.sleep(60) # Prevent multiple triggers

# --- Morning Routine (Mock) ---

if current_time_str == "06:00": # Example sunrise

print("[CARETAKER] Good Morning. Opening Curtains.")

# [HARDWARE] stepper_motor.step(2000)

time.sleep(60)

time.sleep(10) # Check every 10 seconds

modules/sustainer.py (Biosphere):
import time

from modules import comms

# from modules.mcp3008 import MCP3008 # Uncomment on Pi

def run(config, client):

print(">> Sustainer Module: ONLINE")

# adc = MCP3008() # Uncomment on Pi

while True:

# [SIMULATION] Generate fake moisture data

import random

moisture_level = random.randint(20, 80)

# [HARDWARE] Uncomment on Pi

# moisture_level = adc.read(0)

msg = f"SOIL_LEVEL:{moisture_level}"

comms.publish(client, config['mqtt']['topics']['biosphere'], msg)

if moisture_level < 30:

print(f"[SUSTAINER] Soil dry ({moisture_level}%). Watering...")

# [HARDWARE] GPIO.output(pump_pin, GPIO.HIGH)

time.sleep(2)

# [HARDWARE] GPIO.output(pump_pin, GPIO.LOW)

time.sleep(5) # Check every 5 secondsz

The Orchestrator:

main.py:
import multiprocessing

import yaml

import time

import os

from modules import sentinel, caretaker, sustainer, comms

# Load Configuration

def load_config():

with open("config.yaml", "r") as f:

return yaml.safe_load(f)

CONFIG = load_config()

# Wrapper functions for multiprocessing

def start_sentinel():

client = comms.init_mqtt(CONFIG['mqtt']['broker'])

sentinel.run(CONFIG, client)

def start_caretaker():

client = comms.init_mqtt(CONFIG['mqtt']['broker'])

caretaker.run(CONFIG, client)

def start_sustainer():

client = comms.init_mqtt(CONFIG['mqtt']['broker'])

sustainer.run(CONFIG, client)

if __name__ == "__main__":

print("🛡️  INITIALIZING THE FORTRESS OS...")

# Hardware Setup (Mocking GPIO for Simulation)

if CONFIG['system']['mode'] == "PRODUCTION":

import RPi.GPIO as GPIO

GPIO.setmode(GPIO.BCM)

GPIO.setup(CONFIG['gpio']['relay_lock'], GPIO.OUT)

# Setup other pins...

else:

print("⚠️  RUNNING IN SIMULATION MODE (No Hardware IO)")

# Start Processes

processes = []

processes.append(multiprocessing.Process(target=start_sentinel))

processes.append(multiprocessing.Process(target=start_caretaker))

processes.append(multiprocessing.Process(target=start_sustainer))

for p in processes:

p.start()

try:

while True:

time.sleep(1)

except KeyboardInterrupt:

print("\n🛑 SHUTTING DOWN SYSTEMS...")

for p in processes:

p.terminate()

if CONFIG['system']['mode'] == "PRODUCTION":

GPIO.cleanup()

The Dashboard (UI):
dashboard/app.py:
import streamlit as st

import paho.mqtt.client as mqtt

import time

import yaml

# Load Config

with open("config.yaml", "r") as f:

config = yaml.safe_load(f)

# MQTT Setup for Dashboard

def on_message(client, userdata, msg):

# Update session state with new data

topic = msg.topic

payload = msg.payload.decode()

if "security" in topic:

st.session_state['security_status'] = payload

elif "biosphere" in topic:

st.session_state['soil_status'] = payload

client = mqtt.Client("Fortress_Dashboard")

client.on_message = on_message

client.connect(config['mqtt']['broker'], 1883, 60)

client.subscribe(f"{config['mqtt']['topics']['security']}/#")

client.subscribe(f"{config['mqtt']['topics']['biosphere']}/#")

client.loop_start()

# --- UI Layout ---

st.set_page_config(page_title="The Fortress", layout="wide")

st.title("🛡️ The Fortress: Command Center")

# Initialize State

if 'security_status' not in st.session_state:

st.session_state['security_status'] = "SAFE"

if 'soil_status' not in st.session_state:

st.session_state['soil_status'] = "SOIL_LEVEL: --"

# Top Metrics

col1, col2, col3 = st.columns(3)

status_color = "normal" if st.session_state['security_status'] == "SAFE" else "inverse"

col1.metric("System Status", st.session_state['security_status'], delta_color=status_color)

col2.metric("Next Prayer", "Maghrib", "17:45") # Placeholder

col3.metric("Biosphere", st.session_state['soil_status'])

st.markdown("---")

# Controls

c1, c2 = st.columns(2)

with c1:

st.subheader("🔒 Security Override")

if st.button("TRIGGER LOCKDOWN", type="primary"):

client.publish(config['mqtt']['topics']['security'], "LOCKDOWN_MANUAL")

st.error("LOCKDOWN SIGNAL SENT")

with c2:

st.subheader("☀️ Routine Override")

if st.button("Open Curtains"):

client.publish(config['mqtt']['topics']['automation'], "CURTAINS_OPEN")

st.success("Opening...")

# Auto-refresh loop (Streamlit specific trick)

time.sleep(1)

st.rerun()